Metal-Era			February 2026

1600 Airport Road

Waukesha, Wisconsin 53188

Phone	800-558-2162

Fax	800-373-9156

Website	www.metalera.com

Email	info@metalera.com

Guide Specification

Specifier Notes:  This guide specification is written in Construction Specifications Institute (CSI) 3-Part Format in accordance with The CSI Construction Specifications Practice Guide, MasterFormat, SectionFormat, and PageFormat.

This Section must be carefully reviewed and edited by the Architect to meet the requirements of the Project and local building code.  Coordinate this Section with Division 01, other specification sections, and the Drawings.  Delete all Specifier Notes after editing this Section.

Section numbers and titles are based on MasterFormat 2016 Edition.

07 72 29

SLOPED ROOF VENTILATION

Specifier Notes:  This Section covers Metal-Era sloped roof ventilation.  Consult Metal-Era for assistance in editing this Section as required for the Project.

Metal-Era sloped roof ventilation systems are a passive type of ventilation system and are not recommended for roof slopes less than 3/12.

GENERAL

SECTION INCLUDES

Sloped Roof Ventilation:

Specifier Notes:  Edit the following list to include the materials specified in this Section.

Ridge vents.

Vented fasciae.

RELATED REQUIREMENTS

Specifier Notes:  Edit the following list of related sections as required for the Project.  Limit the list to sections with specific information that the reader might expect to find in this Section, but is specified elsewhere.

Section 07 92 00 – Joint Sealants.

REFERENCE STANDARDS

Specifier Notes:  List reference standards used elsewhere in this Section, complete with designations and titles.  Delete reference standards from the following list not used in the edited Section.

Florida Building Code, Testing Application Standard (TAS) 100 (A) – Test Procedure for Wind and Wind Driven Rain Resistance of Discontinuous Roof Systems.

Miami-Dade County, Florida Notice of Acceptance (NOA).

PREINSTALLATION MEETINGS

Specifier Notes:  Edit the Preinstallation Meetings article as required for the Project.  Delete article if not required.

Convene preinstallation meeting [1 week]  [2 weeks] before start of installation of sloped roof ventilation.

Require attendance of parties directly affecting Work of this Section, including Contractor, Architect, installer, and manufacturer’s representative.

Review the Following:

Materials.

Installation.

Adjusting.

Cleaning.

Protection.

Coordination with other Work.

SUBMITTALS

Specifier Notes:  Edit the Submittals article as required for the Project.  Delete submittals not required.

Comply with Division 01.

Product Data:  Submit manufacturer’s product data, including installation instructions.

Shop Drawings:  Submit manufacturer’s shop drawings, including plans, elevations, sections, and details, indicating dimensions, materials, components, fasteners, finish, and accessories.

Samples:  Submit manufacturer’s sample of sloped roof ventilation.

Sample Length:  Minimum 5-1/2 inches (140 mm).

Color Samples:  Submit manufacturer’s color samples of sloped roof ventilation, consisting of complete color chart representing manufacturer’s full range of available colors.

Specifier Notes:  Include the following sentence as required for the Project.  Delete if not required.

Submit metal chips of specific colors as requested by the Architect.

Manufacturer’s Certification:  Submit manufacturer’s certification that sloped roof ventilation complies with specified requirements and is suitable for intended application.

Warranty Documentation:  Submit manufacturer’s standard warranty.

QUALITY ASSURANCE

Manufacturer’s Qualifications:  Manufacturer regularly engaged in the manufacturing of sloped roof ventilation of similar type to that specified for a minimum of 10 years.

Installer's Qualifications:

Installer regularly engaged in installation of sloped roof ventilation of similar type to that specified for a minimum of 5 years.

Use persons trained for installation of sloped roof ventilation of similar type to that specified following manufacturer’s installation instructions.

Testing:  Meet specified testing requirements.

DELIVERY, STORAGE, AND HANDLING

Delivery Requirements:  Deliver materials to site in manufacturer’s original, unopened containers and packaging.

Storage and Handling Requirements:

Store and handle sloped roof ventilation in accordance with manufacturer’s instructions.

Inspect and report damage or other issues to manufacturer within 72 hours of receipt of materials.

Keep sloped roof ventilation in manufacturer’s original, unopened containers and packaging until installation.

Store sloped roof ventilation in clean, dry area indoors.

Do not store sloped roof ventilation directly on floor or ground.

Protect sloped roof ventilation and finish during storage, handling, and installation to prevent damage.

WARRANTY

Specifier Notes:  Include warranty period for the sloped roof ventilation and finish specified.  Edit the following as required for the Project.

In addition to the limited 30-year finish warranty, other finish warranty periods are available.  Consult Metal-Era for warranty period information for custom post-coated Kynar 500 colors and for anodized finishes.

Warranty Period, Product:

Metal-Era “Hi-Perf” Ventilation Systems:  20 years or lifetime of roof on which it was originally installed, whichever comes first, and when intake and exhaust ventilation is designed by manufacturer as a complete system.

Wind:  Maximum 130 mph.

Other Products:  Five-year workmanship warranty covering replacement or repair of products that are defective in material or workmanship.

Warranty Period, Finish:  Limited 30-year warranty for prefinished coil-coated steel and aluminum coated with Kynar 500 standard colors covering fade, chalk, and film integrity.

PRODUCTS

MANUFACTURERS

Manufacturer:  Metal-Era, 1600 Airport Road, Waukesha, Wisconsin 53188.  Phone 800-558-2162.  Fax 800-373-9156.  www.metalera.com.  info@metalera.com.

Specifier Notes:  Specify if substitutions will be permitted.

Substitutions:  [Not permitted]  [Comply with Division 01].

Specifier Notes:  Specify Metal-Era sloped roof ventilation as required for the Project.  Delete materials not required.

Consult Metal-Era for custom capabilities.

RIDGE VENTS

Ridge Vents:  Metal-Era “Hi-Perf” ridge vent, slope-to-slope, shingled version.

Water Infiltration Test, TAS 100 (A):  Passed.

Specifier Notes:  Specify ridge vent cover material.  Delete cover materials not required.  Consult Metal-Era for information regarding other materials.

Ridge Vent Cover Material:  [24-gauge (0.65-mm) galvanized steel]  [0.040-inch (1.01-mm) aluminum]  [0.050-inch (1.27-mm) aluminum]  [0.063-inch (1.60-mm) aluminum]  [____________].

Formed Lengths:  12’-0” (3.65 m), with slotted fastening holes.

Cover Plates:  At each joint, 8 inches (203 mm) wide.

Continuous Z-Brackets:  20-gauge (0.91-mm) galvanized steel.

Intermittent Spacers:  0.050-inch (1.27-mm) aluminum, 12 inches (305 mm) on center.

Expanded Metal Support Screens:  0.050-inch (1.27-mm) aluminum.

Specifier Notes:  Specify style of endcaps.

Endcaps:  [Style A]  [Style B].

Specifier Notes:  Deflectors are required if roof pitch is less than 4/12.

Deflectors:

Roof Pitch:  Less than 4/12.

Formed Lengths:  12’-0” (3.65 m).

Deflector Baffles:  At deflector joints, 3-1/2 inches (89 mm) wide.

Continuous Z-Brackets, Intermittent Spacers, and Expanded Metal Support Screens:  Factory assembled.

Ridge Vents:  Metal-Era “Hi-Perf” ridge vent, sloped roof meets high wall version.

Specifier Notes:  Specify ridge vent cover material.  Delete cover materials not required.  Consult Metal-Era for information regarding other materials.

Ridge Vent Cover Material:  [24-gauge (0.65-mm) galvanized steel]  [0.040-inch (1.01-mm) aluminum]  [0.050-inch (1.27-mm) aluminum]  [0.063-inch (1.60-mm) aluminum]  [____________].

Formed Lengths:  12’-0” (3.65 m), with slotted fastening holes.

Cover Plates:  At each joint, 8 inches (203 mm) wide.

Continuous Z-Brackets:  20-gauge (0.91-mm) galvanized steel.

Expanded Metal Support Screens:  0.050-inch (1.27-mm) aluminum.

Specifier Notes:  Specify style of endcaps.

Endcaps:  [Style A]  [Style B].

Specifier Notes:  Deflectors are required if roof pitch is less than 4/12.

Deflectors:

Roof Pitch:  Less than 4/12.

Formed Lengths:  12’-0” (3.65 m).

Deflector Baffles:  At deflector joints, 3-1/2 inches (89 mm) wide.

VENTED FASCIAE

Vented Fasciae:  Metal-Era “Hi-Perf” vented fascia, vented-nailbase version.

Specifier Notes:  Specify fascia cover material.  Delete cover materials not required.  Consult Metal-Era for information regarding other materials.

Fascia Cover Material:  [24-gauge (0.65-mm) galvanized steel]  [22-gauge (0.81-mm) galvanized steel]  [0.040-inch (1.01-mm) aluminum]  [0.050-inch (1.27-mm) aluminum]  [0.063-inch (1.60-mm) aluminum]  [____________].

Formed Lengths:  12’-0” (3.65 m), with slotted fastening holes.

Concealed Splice Plates:  At each joint, 8 inches (203 mm) wide.

Vertical Z-Brackets:  0.050-inch (1.27-mm) aluminum, 12 inches (305 mm) on center.

Backer:  24-gauge (0.65-mm) galvanized steel.

Factory assembled to vertical Z-brackets and continuous roof flange.

Perforated Closures:  24-gauge (0.65-mm) galvanized steel.

Net Free Area:  54 percent.

Specifier Notes:  Specify color of perforated closures.

Color:  [Bone White]  [Dark Bronze]  [Sandstone]  [Slate Gray].

Continuous Roof Flange:  24-gauge (0.65-mm) galvanized steel.

Specifier Notes:  Specify top of nailbase to bottom of fascia dimension.  Consult Metal-Era for information regarding other dimensions.

Top of Nailbase to Bottom of Fascia:  [6-inch (152-mm) coverage]  [8-inch (203-mm) coverage]  [10-inch (254-mm) coverage]  [12-inch (305-mm) coverage]  [14-inch (357-mm) coverage]  [16-inch (406-mm) coverage]  [____________].

Vented Fasciae:  Metal-Era “Hi-Perf” vented fascia, attic-vented version.

Specifier Notes:  Specify fascia cover material.  Delete cover materials not required.  Consult Metal-Era for information regarding other materials.

Fascia Cover Material:  [24-gauge (0.65-mm) galvanized steel]  [22-gauge (0.81-mm) galvanized steel]  [0.040-inch (1.01-mm) aluminum]  [0.050-inch (1.27-mm) aluminum]  [0.063-inch (1.60-mm) aluminum]  [____________].

Formed Lengths:  12’-0” (3.65 m), with slotted fastening holes.

Concealed Splice Plates:  At each joint, 8 inches (203 mm) wide.

Vertical Z-Brackets:  0.050-inch (1.27-mm) aluminum, 12 inches (305 mm) on center.

Backer:  24-gauge (0.65-mm) galvanized steel.

Factory assembled to vertical Z-brackets and continuous roof flange.

Perforated Closures:  24-gauge (0.65-mm) galvanized steel.

Net Free Area:  54 percent.

Specifier Notes:  Specify color of perforated closures.

Color:  [Bone White]  [Dark Bronze]  [Sandstone]  [Slate Gray].

Continuous Roof Flange:  24-gauge (0.65-mm) galvanized steel.

Specifier Notes:  Specify top of sheathing to bottom of fascia dimension.  Consult Metal-Era for information regarding other dimensions.

Top of Sheathing to Bottom of Fascia:  [6-inch (152-mm) coverage]  [8-inch (203-mm) coverage]  [10-inch (254-mm) coverage]  [12-inch (305-mm) coverage]  [14-inch (357-mm) coverage]  [16-inch (406-mm) coverage]  [____________].

FINISHES

Specifier Notes:  Specify finish for the materials.  Delete finish not required.  Consult Metal-Era for description of the finishes available for each cover material and for information regarding other finishes.

Finish:  [Hylar 5000/Kynar 500]  [Coil anodized]  [____________].

Specifier Notes:  Specify color.  Consult Metal-Era for standard and custom colors.

Color:  ____________.

ACCESSORIES

Specifier Notes:  Delete accessories not required.

Joint Sealants:  Specified in Section 07 92 00.

Fasteners:  Appropriate for intended substrate.

EXECUTION

EXAMINATION

Examine areas to receive sloped roof ventilation.

Verify surfaces to support sloped roof ventilation are clean, dry, straight, secure, and of proper dimensions.

Notify Architect of conditions that would adversely affect installation.

Do not begin installation until unacceptable conditions are corrected.

INSTALLATION

Install sloped roof ventilation in accordance with manufacturer’s instructions at locations indicated on the Drawings.

Remove protective vinyl film immediately before installation.

Install sloped roof ventilation watertight.

Install sloped roof ventilation to allow for thermal movement.

Joint Sealants:  Apply joint sealants in accordance with manufacturer’s instructions.

ADJUSTING

Repair minor damages to finish in accordance with manufacturer’s instructions and as approved by Architect.

Remove and replace with new material, damaged components that cannot be successfully repaired, as determined by Architect.

CLEANING

Clean sloped roof ventilation promptly after installation in accordance with manufacturer’s instructions.

Do not use harsh cleaning materials or methods that could damage finish.

PROTECTION

Protect installed sloped roof ventilation to ensure that, except for normal weathering, materials will be without damage or deterioration at time of Substantial Completion.

END OF SECTION